Quarterly Maintenance Checklist - Rotating Equipment, Area C

| Document No. | FTP-OPS-CHK-031 | Revision | 3.2 |
| --- | --- | --- | --- |
| Checklist Period | Q3 2026 (Jul-Sep) | Department | Operations / Mechanical Maintenance |
| Area / Location | Area C - Rotating Equipment Bay | Frequency | Weekly (13 checks per quarter) |
| Prepared By | R. Iyer, Head - Operations | Approved By | Plant Manager, FortTrace Plant Operations |

# 1. Purpose

This checklist records the weekly preventive maintenance inspection of rotating equipment installed in Area C (Rotating Equipment Bay) for the third quarter of 2026. It covers routine condition checks on pumps, the process air compressor, the cooling air blower, and the agitator drive train, in accordance with the Area C preventive maintenance schedule.

# 2. Weekly Inspection Task List

| S.No | Equipment Tag | Inspection Task | Frequency | Acceptance Standard | Status (P/F) | Remarks |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | P-101A | Check pump bearing housing temperature (DE / NDE) | Weekly | ≤ 70°C (per FTP-MNT-STD-004) |  |  |
| 2 | P-101B | Check pump bearing housing temperature (DE / NDE) | Weekly | ≤ 70°C (per FTP-MNT-STD-004) |  |  |
| 3 | P-101A/B | Inspect for shaft seal / gland packing leakage | Weekly | No visible leakage beyond drip tray |  |  |
| 4 | P-101A/B | Vibration check - overall velocity (RMS) | Weekly | ≤ 4.5 mm/s (ISO 10816-3, Zone B) |  |  |
| 5 | C-201 | Check compressor discharge pressure vs. rated | Weekly | Within ±5% of design (12.5 barg) |  |  |
| 6 | C-201 | Inspect inter-stage cooler for fouling / leaks | Weekly | No fouling, no visible leaks |  |  |
| 7 | C-201 | Check lube oil level and condition (sight glass) | Weekly | Between MIN-MAX marks, no discoloration |  |  |
| 8 | B-305 | Check blower belt tension and alignment | Weekly | Deflection per OEM manual ±10% |  |  |
| 9 | B-305 | Inspect motor cooling fan and air intake screen | Weekly | Free of dust/debris blockage |  |  |
| 10 | AG-410 | Check agitator gearbox oil temperature | Weekly | ≤ 80°C (per OEM datasheet) |  |  |
| 11 | AG-410 | Inspect coupling guard integrity | Weekly | Guard intact, all fasteners secure |  |  |
| 12 | P-101A/B | Check motor winding temperature (RTD readout) | Weekly | ≤ Class F limit (155°C) |  |  |
| 13 | C-201 | Verify safety relief valve seal / tag intact | Weekly | Tamper seal unbroken, tag legible |  |  |
| 14 | B-305 / AG-410 | General housekeeping - area free of oil spills | Weekly | No slip hazards, area clean |  |  |

# 3. Instructions for Use

- Enter P (Pass) or F (Fail) in the Status column for each task at every weekly round.

- Any Fail entry must be accompanied by a Remarks note and a corresponding maintenance notification (MN) number.

- Vibration readings above the ISO 10816-3 Zone B threshold shall be reported to the Predictive Maintenance group within 24 hours.

- Completed checklists shall be scanned and uploaded to the FortTrace document management system within 2 working days of each round.

# 4. Weekly Sign-Off Log (Q3 2026)

| Wk | Date | Technician | Shift | Shift In-Charge Sign-Off |
| --- | --- | --- | --- | --- |
| 1 | 06-Jul-2026 |  |  |  |
| 2 | 13-Jul-2026 |  |  |  |
| 3 | 20-Jul-2026 |  |  |  |
| 4 | 27-Jul-2026 |  |  |  |
| 5 | 03-Aug-2026 |  |  |  |
| 6 | 10-Aug-2026 |  |  |  |
| 7 | 17-Aug-2026 |  |  |  |
| 8 | 24-Aug-2026 |  |  |  |
| 9 | 31-Aug-2026 |  |  |  |
| 10 | 07-Sep-2026 |  |  |  |
| 11 | 14-Sep-2026 |  |  |  |
| 12 | 21-Sep-2026 |  |  |  |
| 13 | 28-Sep-2026 |  |  |  |